Klicni center

Vstavi tloris (call centre.dwg) in ustvari prostor

Določite vidne naloge in lokacije in preverite kakšno osvetljenost morate zagotoviti. Izberite svetilke in jih vstavite. Izvedite tudi vse potrebne izračune.

Ideje za svetilke:

- Intra Minus S: https://www.intra-lighting.com/family-child?family=MinusS

- Ledluks Ilo D/I (http://www.ledluks.com/products/ilo-2/ )?

Namig:

https://evo.support-en.dial.de/support/solutions/articles/9000115442-visual-task-areas

https://evo.support-en.dial.de/support/solutions/articles/9000121058-guideline-for-workplaces-asr-a3-4

https://evo.support-en.dial.de/support/solutions/articles/9000115446-areas-of-activity

https://www.fagerhult.com/knowledge-hub/EN-12464-1/Principles-for-the-working-area/

| Illuminance in the working area | Illuminance within the immediate surroundings | Illuminance within the peripheral surroundings in relation to the immediate surroundings |
| --- | --- | --- |
| ≥ 750 | 500 | 1/3 |
| 500 | 300 | 1/3 |
| 300 | 200 | 1/3 |
| 200 | 150 | 1/3 |
| 150 | Eworking area | 1/3 |
| 100 | Eworking area | 1/3 |
| ≤ 50 | Eworking area | 1/3 |